~Weekly Review and practice Practical:-

- On Cisco Packet Tracer, create a network with three switches and assign at least two different VLANs (e.g., VLAN 10 for 'Sales', VLAN 20 for 'Support'). Connect two PCs to each switch and assign each PC to the correct VLAN based on department.

Network Setup:

Devices:

- 3 × Cisco 2960 switches

- 6 × PCs

- VLAN 10 = Sales

- VLAN 20 = Support

1. Connect PCs:

| Switch | PC | Port | VLAN |
| --- | --- | --- | --- |
| Switch1 | PC1 | Fa0/2 | VLAN 10 – Sales |
| Switch1 | PC2 | Fa0/3 | VLAN 20 – Support |
| Switch2 | PC3 | Fa0/2 | VLAN 10 – Sales |
| Switch2 | PC4 | Fa0/3 | VLAN 20 – Support |
| Switch3 | PC5 | Fa0/2 | VLAN 10 – Sales |
| Switch3 | PC6 | Fa0/3 | VLAN 20 – Support |

2. Create VLANs on all switches:

Run on Switch1, Switch2, and Switch3:

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

3. Configure Switch1:

interface fa0/2

switchport mode access

switchport access vlan 10

exit

interface fa0/3

switchport mode access

switchport access vlan 20

exit

end

4. Configure Switch2:

interface fa0/2

switchport mode access

switchport access vlan 10

exit

interface fa0/3

switchport mode access

switchport access vlan 20

exit

end

5. Configure Switch3:

interface fa0/2

switchport mode access

switchport access vlan 10

exit

interface fa0/3

switchport mode access

switchport access vlan 20

exit

end

6. Verify:

On each switch:

show vlan brief

Expected:

VLAN 10  Sales    Fa0/2

VLAN 20  Support  Fa0/3

Result: Each switch has one Sales PC in VLAN 10 and one Support PC in VLAN 20.

- Configure trunk ports between the switches to allow VLAN traffic to pass through. Verify trunking by using the show interfaces trunk command and submit a screenshot of the output.

1. Connect the switches:

Use Copper Cross-Over cables:

Switch1 Fa0/1 ───── Fa0/1 Switch2

Switch2 Fa0/5 ───── Fa0/1 Switch3

2. Configure Switch1:

enable

configure terminal

interface fa0/1

switchport mode trunk

no shutdown

end

3. Configure Switch2:

enable

configure terminal

interface fa0/1

switchport mode trunk

no shutdown

exit

interface fa0/5

switchport mode trunk

no shutdown

end

4. Configure Switch3:

enable

configure terminal

interface fa0/1

switchport mode trunk

no shutdown

end

5. Verify trunking:

Run this command on all three switches:

show interfaces trunk

You should see the trunk ports listed, with VLANs 10 and 20 allowed.

Example output:

Port        Mode         Encapsulation  Status       Native vlan

Fa0/1       on           802.1q         trunking     1

Port        Vlans allowed on trunk

Fa0/1       10,20

- Set up a Layer 3 device (Router-on-a-Stick using a router or a multilayer switch) to enable inter-VLAN routing between VLAN 10 and VLAN 20. Assign appropriate sub-interfaces and IP addresses for each VLAN.

Router-on-a-Stick Configuration:

Use a router connected to Switch1. The switch port connected to the router must be a trunk.

1. Configure Switch1 trunk:

Assume the router is connected to Fa0/6:

enable

configure terminal

interface fa0/6

switchport mode trunk

no shutdown

end

2. Configure Router:

Assume the router uses GigabitEthernet0/0:

enable

configure terminal

interface gigabitEthernet 0/0

no shutdown

exit

3. VLAN 10 – Sales:

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

4. VLAN 20 – Support:

interface gigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

end

5. Configure PC gateways:

Sales PC:

IP: 192.168.10.10

Mask: 255.255.255.0

Gateway: 192.168.10.1

Support PC:

IP: 192.168.20.10

Mask: 255.255.255.0

Gateway: 192.168.20.1

6. Verify:

On the router:

show ip interface brief

You should see:

G0/0.10   192.168.10.1   up   up

G0/0.20   192.168.20.1   up   up

Test from the Sales PC:

ping 192.168.20.10

✅ If the configuration is correct, the ping should succeed.

Result: VLAN 10 (Sales) and VLAN 20 (Support) can now communicate through the router using G0/0.10 and G0/0.20.

- Test connectivity by pinging between a PC in VLAN 10 and a PC in VLAN 20. If pings fail, troubleshoot and fix the configuration so that inter-VLAN communication works.<br><br><em><strong>Hint:</strong> Check sub-interface IPs, trunk configuration, and default gateways on PCs.</em>

1. Test from VLAN 10 PC:

Open PC in VLAN 10 → Desktop → Command Prompt:

ping 192.168.20.10

Expected:

Reply from 192.168.20.10

2. If ping fails, check these settings:

Router sub-interfaces:

show ip interface brief

Check:

G0/0.10 → 192.168.10.1 → up/up

G0/0.20 → 192.168.20.1 → up/up

Router configuration:

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface g0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

3. Check the switch trunk:

On the switch:

show interfaces trunk

The router-connected port should be trunking and carry VLANs 10 and 20.

If required:

interface fa0/6

switchport mode trunk

no shutdown

4. Check PC default gateways:

VLAN 10 PC:

IP:      192.168.10.10

Mask:    255.255.255.0

Gateway: 192.168.10.1

VLAN 20 PC:

IP:      192.168.20.10

Mask:    255.255.255.0

Gateway: 192.168.20.1

5. Test again:

From VLAN 10 PC:

ping 192.168.20.10

✅ Ping should now succeed.

From VLAN 20 PC:

ping 192.168.10.10

✅ Ping should also succeed.

Conclusion: Inter-VLAN communication works when the router sub-interfaces have the correct VLAN IDs/IPs, the switch-to-router link is a trunk, and each PC uses the correct sub-interface as its default gateway.

THE END……