~Day-5:

- Using Cisco Packet Tracer, create a network topology with three routers in two OSPF areas (Area 0 and Area 1), and assign appropriate IP addresses to all interfaces.

Here is a simple 3-router OSPF topology with 2 areas that you can directly build in Cisco Packet Tracer.

1. Topology:

PC1 ── SW1 ── R1 ───── R2 ───── R3 ── SW2 ── PC2

Area 0       Area 1

- R1–R2: Area 0

- R2–R3: Area 1

- R2 is the ABR (Area Border Router) because it connects Area 0 and Area 1.

2. IP Addressing:

| Device | Interface | IP Address | Area |
| --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1/24 | Area 0 |
| R1 | G0/1 | 10.0.0.1/30 | Area 0 |
| R2 | G0/0 | 10.0.0.2/30 | Area 0 |
| R2 | G0/1 | 10.0.0.5/30 | Area 1 |
| R3 | G0/0 | 10.0.0.6/30 | Area 1 |
| R3 | G0/1 | 192.168.2.1/24 | Area 1 |
| PC1 | NIC | 192.168.1.10/24 | — |
| PC2 | NIC | 192.168.2.10/24 | — |

Gateway:

- PC1 → 192.168.1.1

- PC2 → 192.168.2.1

3. R1 Configuration:

enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

exit

4. R2 Configuration:

enable

configure terminal

interface g0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.0.5 255.255.255.252

no shutdown

exit

router ospf 1

router-id 2.2.2.2

network 10.0.0.0 0.0.0.3 area 0

network 10.0.0.4 0.0.0.3 area 1

exit

5. R3 Configuration:

enable

configure terminal

interface g0/0

ip address 10.0.0.6 255.255.255.252

no shutdown

exit

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

router ospf 1

router-id 3.3.3.3

network 10.0.0.4 0.0.0.3 area 1

network 192.168.2.0 0.0.0.255 area 1

exit

6. Configure PCs:

PC1:

- IP: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PC2:

- IP: 192.168.2.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.2.1

7. Verify OSPF:

On each router:

show ip ospf neighbor

You should see OSPF neighbors in FULL state.

Check learned routes:

show ip route ospf

On R1, you should see the 192.168.2.0/24 network learned through R2.

On R3, you should see the 192.168.1.0/24 network learned through R2.

Finally, test:

PC1 → ping 192.168.2.10

If the ping succeeds, Area 0 and Area 1 are successfully communicating through R2.

- Configure OSPF on each router in your topology so that Area 0 acts as the backbone and Area 1 connects through one of the routers; verify OSPF neighbor relationships using the 'show ip ospf neighbor' command.

Use the same 3-router topology. R2 will act as the ABR, connecting Area 0 (backbone) and Area 1.

1. Configure R1 — Area 0:

enable

configure terminal

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

exit

2. Configure R2 — ABR:

R2 has one interface in Area 0 and one in Area 1.

enable

configure terminal

router ospf 1

router-id 2.2.2.2

network 10.0.0.0 0.0.0.3 area 0

network 10.0.0.4 0.0.0.3 area 1

exit

3. Configure R3 — Area 1:

enable

configure terminal

router ospf 1

router-id 3.3.3.3

network 10.0.0.4 0.0.0.3 area 1

network 192.168.2.0 0.0.0.255 area 1

exit

4. Verify OSPF neighbors:

On R1:

show ip ospf neighbor

Expected neighbor:

Neighbor ID     State       Address

2.2.2.2         FULL        10.0.0.2

On R2:

show ip ospf neighbor

Expected:

1.1.1.1         FULL        10.0.0.1

3.3.3.3         FULL        10.0.0.6

On R3:

show ip ospf neighbor

Expected:

2.2.2.2         FULL        10.0.0.5

5. Verify OSPF areas:

show ip ospf

You should see:

- R1: Area 0

- R2: Area 0 and Area 1 → ABR

- R3: Area 1

Result

AREA 0 (BACKBONE)

R1 ───────────── R2

│

│

AREA 1

│

R3

The FULL state confirms that OSPF neighbor adjacency has successfully formed between the routers. R2 connects Area 1 to the Area 0 backbone.

- Add two PCs to different areas in your OSPF network, assign them IP addresses, and test connectivity by pinging from one PC to the other.

Using the same topology, add PC1 in Area 0 and PC2 in Area 1.

1. Add PCs:

Connect:

PC1 ── SW1 ── R1 ───── R2 ───── R3 ── SW2 ── PC2

Area 0       Area 1

2. Configure PC1 — Area 0:

Go to PC1 → Desktop → IP Configuration:

| Setting | Value |
| --- | --- |
| IP Address | 192.168.1.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.1 |

PC1 belongs to the LAN connected to R1 (Area 0).

3. Configure PC2 — Area 1:

Go to PC2 → Desktop → IP Configuration:

| Setting | Value |
| --- | --- |
| IP Address | 192.168.2.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.2.1 |

PC2 belongs to the LAN connected to R3 (Area 1).

4. Check OSPF routes:

On R1:

show ip route ospf

You should see the Area 1 network:

O    192.168.2.0/24

On R3:

show ip route ospf

You should see:

O    192.168.1.0/24

5. Test connectivity:

On PC1 → Desktop → Command Prompt:

ping 192.168.2.10

Expected result:

Reply from 192.168.2.10: bytes=32 time<1ms TTL=...

Reply from 192.168.2.10: bytes=32 time<1ms TTL=...

Reply from 192.168.2.10: bytes=32 time<1ms TTL=...

Reply from 192.168.2.10: bytes=32 time<1ms TTL=...

You can also test in the opposite direction from PC2:

ping 192.168.1.10

Result:

PC1 (Area 0) → R1 → R2 (ABR) → R3 → PC2 (Area 1)

If the ping receives replies, it confirms that OSPF is successfully routing traffic between Area 0 and Area 1.

- Simulate a router failure by shutting down the interface connecting Area 1 to Area 0, then use 'show ip route' and 'show ip ospf' commands to observe how OSPF reconverges and restores connectivity after re-enabling the interface.<br><br><em><strong>Hint:</strong> Track the time it takes for OSPF to update the routing tables after the failure and recovery.</em>

Use the same 3-router topology where R2 is the ABR:

PC1 ─ R1 ───── R2 ───── R3 ─ PC2

Area 0    │    Area 1

↑

ABR

1. Check the network before failure:

On R2, verify the interfaces:

show ip interface brief

Then check OSPF:

show ip ospf neighbor

show ip route

show ip ospf

You should have an OSPF neighbor with R3 (3.3.3.3) and routes to the Area 1 network.

From PC1, test:

ping 192.168.2.10

The ping should succeed.

2. Simulate the failure:

The R2 interface connecting to R3 is:

R2 G0/1 = 10.0.0.5

R3 G0/0 = 10.0.0.6

On R2:

enable

configure terminal

interface g0/1

shutdown

exit

The Area 1 connection is now down.

3. Observe OSPF reconvergence:

Immediately check:

show ip ospf neighbor

The R3 neighbor should disappear or no longer be in the FULL state.

Now check:

show ip route

The route to:

192.168.2.0/24

should be removed because R2 no longer has a path to Area 1.

Also check:

show ip ospf

This lets you observe the OSPF process and area information after the topology change.

Test from PC1:

ping 192.168.2.10

The ping should fail while the connection is down.

4. Restore the connection:

On R2:

configure terminal

interface g0/1

no shutdown

exit

Wait a few seconds for OSPF to establish the adjacency again.

Check:

show ip ospf neighbor

You should again see R3:

Neighbor ID     State

3.3.3.3         FULL

Then:

show ip route

The route to 192.168.2.0/24 should return.

5. Test connectivity again:

From PC1:

ping 192.168.2.10

You should receive replies again.

6. Record the reconvergence time:

For your practical record, note the approximate time between:

Failure:

R2(config-if)# shutdown

and

Recovery of routing:

show ip ospf neighbor

show ip route

Then repeat after:

no shutdown

A simple observation table:

| Event | Observation |
| --- | --- |
| Before shutdown | R2–R3 neighbor = FULL |
| Interface shutdown | R2–R3 adjacency goes down |
| After failure | Area 1 route disappears |
| PC1 → PC2 during failure | Ping fails |
| Interface re-enabled | OSPF adjacency reforms |
| After reconvergence | Area 1 route returns |
| PC1 → PC2 after recovery | Ping succeeds |

Conclusion: When the R2–R3 link fails, OSPF detects the topology change and removes the unreachable Area 1 route. After the interface is re-enabled, OSPF reforms the neighbor adjacency, updates the routing table, and restores connectivity.

THE END……….