~inter-VLAN Routing Basic Theory :-

- In Cisco Packet Tracer, create three VLANs named Foodies, Shoppers, and Streamers to represent users of Zomato, Flipkart, and Spotify, and assign at least one PC to each VLAN.

Here's a simple Cisco Packet Tracer practical for creating three VLANs representing Zomato, Flipkart, and Spotify users.

1. Create the topology:

Add:

- 1 × Cisco 2960 Switch

- 3 × PCs

Connect:

PC1 ─── Fa0/2 ┐

PC2 ─── Fa0/3 ├── Switch

PC3 ─── Fa0/4 ┘

Use Copper Straight-Through cables.

2. Create the three VLANs:

Open Switch → CLI:

enable

configure terminal

vlan 10

name Foodies

exit

vlan 20

name Shoppers

exit

vlan 30

name Streamers

exit

3. Assign PC1 to Foodies:

interface fastethernet 0/2

switchport mode access

switchport access vlan 10

no shutdown

exit

4. Assign PC2 to Shoppers:

interface fastethernet 0/3

switchport mode access

switchport access vlan 20

no shutdown

exit

5. Assign PC3 to Streamers:

interface fastethernet 0/4

switchport mode access

switchport access vlan 30

no shutdown

exit

end

6. Configure IP addresses:

Go to PC → Desktop → IP Configuration.

| PC | Application | VLAN | IP Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| PC1 | Zomato | 10 – Foodies | 192.168.10.10 | 255.255.255.0 |
| PC2 | Flipkart | 20 – Shoppers | 192.168.20.10 | 255.255.255.0 |
| PC3 | Spotify | 30 – Streamers | 192.168.30.10 | 255.255.255.0 |

No default gateway is required for this basic VLAN demonstration.

7. Verify the VLANs:

On the switch:

show vlan brief

Expected result:

VLAN   Name         Status    Ports

10     Foodies      active    Fa0/2

20     Shoppers     active    Fa0/3

30     Streamers    active    Fa0/4

8. Test connectivity:

From PC1:

ping 192.168.20.10

❌ Should fail because PC1 is in VLAN 10 and PC2 is in VLAN 20.

ping 192.168.30.10

❌ Should fail because PC1 is in VLAN 10 and PC3 is in VLAN 30.

Final topology:

┌───────────────┐

PC1 (Zomato) ───┤ Fa0/2 VLAN 10 │

│    FOODIES    │

│               │

PC2 (Flipkart) ─┤ Fa0/3 VLAN 20 │

│    SHOPPERS   │

│               │

PC3 (Spotify) ──┤ Fa0/4 VLAN 30 │

│   STREAMERS   │

└───────────────┘

SWITCH

Conclusion: The three groups are separated into different VLANs. Foodies (Zomato), Shoppers (Flipkart), and Streamers (Spotify) are isolated at Layer 2, so their PCs cannot communicate directly without inter-VLAN routing.

- Configure a router-on-a-stick setup in Packet Tracer so that PCs in the Foodies, Shoppers, and Streamers VLANs can all ping each other using sub-interfaces on the router (e.g., GigabitEthernet0/0.10, .20, .30).

Since your VLANs are already Foodies (10), Shoppers (20), and Streamers (30), you can use Router-on-a-Stick to allow communication between all three VLANs.

1. Topology:

Add 1 router and connect it to the switch:

PC1 ── Fa0/2 ── VLAN 10 ┐

PC2 ── Fa0/3 ── VLAN 20 ├── Switch ── Fa0/1 ── G0/0 Router

PC3 ── Fa0/4 ── VLAN 30 ┘

Use a Copper Straight-Through cable between the switch and router.

2. Configure the Switch trunk:

The switch port connected to the router must be a trunk.

If the router is connected to Fa0/1:

enable

configure terminal

interface fastethernet 0/1

switchport mode trunk

no shutdown

end

Verify:

show interfaces trunk

3. Configure Router sub-interfaces:

Open Router → CLI.

First enable the physical interface:

enable

configure terminal

interface gigabitethernet 0/0

no shutdown

exit

VLAN 10 — Foodies

interface gigabitethernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

VLAN 20 — Shoppers

interface gigabitethernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

VLAN 30 — Streamers

interface gigabitethernet 0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

end

Save the configuration:

copy running-config startup-config

Press Enter when asked for the destination filename.

4. Configure PC IP addresses and gateways:

PC1 — Foodies

Desktop → IP Configuration

IP Address:     192.168.10.10

Subnet Mask:    255.255.255.0

Default Gateway: 192.168.10.1

PC2 — Shoppers

IP Address:     192.168.20.10

Subnet Mask:    255.255.255.0

Default Gateway: 192.168.20.1

PC3 — Streamers

IP Address:     192.168.30.10

Subnet Mask:    255.255.255.0

Default Gateway: 192.168.30.1

5. Test inter-VLAN communication:

From PC1, open Command Prompt:

ping 192.168.20.10

✅ PC1 → PC2 should succeed.

ping 192.168.30.10

✅ PC1 → PC3 should succeed.

From PC2:

ping 192.168.10.10

✅ Should succeed.

From PC3:

ping 192.168.20.10

✅ Should succeed.

6. Verify the router:

Run:

show ip interface brief

You should see something similar to:

Interface              IP-Address      Status

GigabitEthernet0/0     unassigned      up

GigabitEthernet0/0.10  192.168.10.1    up

GigabitEthernet0/0.20  192.168.20.1    up

GigabitEthernet0/0.30  192.168.30.1    up

You can also check the routing table:

show ip route

The three VLAN networks should appear as directly connected.

Final result:

Router

G0/0 │

│

┌────────┴────────┐

│  Router-on-a-   │

│      Stick      │

└────────┬────────┘

│ 802.1Q Trunk

│

Switch

┌────────┼────────┐

│        │        │

VLAN 10  VLAN 20  VLAN 30

Foodies  Shoppers Streamers

│        │        │

PC1      PC2      PC3

Conclusion: The router uses sub-interfaces G0/0.10, G0/0.20, and G0/0.30 to route traffic between VLANs. Therefore, PCs in Foodies, Shoppers, and Streamers can communicate with and ping each other.

- Assign IP addresses to each PC in your VLANs and set their default gateways to the corresponding router sub-interface IPs to enable inter-VLAN communication.

Since your Router-on-a-Stick setup is already configured, assign these IP addresses to the PCs.

1. PC1 — Foodies (VLAN 10):

Go to PC1 → Desktop → IP Configuration:

IP Address:       192.168.10.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.10.1

Gateway 192.168.10.1 is the router's G0/0.10 sub-interface.

2. PC2 — Shoppers (VLAN 20):

IP Address:       192.168.20.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.20.1

Gateway 192.168.20.1 is the router's G0/0.20 sub-interface.

3. PC3 — Streamers (VLAN 30):

IP Address:       192.168.30.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.30.1

Gateway 192.168.30.1 is the router's G0/0.30 sub-interface.

IP Address Summary:

| PC | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 – Foodies | 10 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC2 – Shoppers | 20 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| PC3 – Streamers | 30 | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |

4. Test inter-VLAN communication

From PC1 → Desktop → Command Prompt:

ping 192.168.20.10

ping 192.168.30.10

From PC2:

ping 192.168.10.10

ping 192.168.30.10

From PC3:

ping 192.168.10.10

ping 192.168.20.10

✅ All pings should succeed.

Conclusion: Each PC uses its VLAN's router sub-interface as its default gateway. The router then forwards traffic between VLAN 10, VLAN 20, and VLAN 30, enabling inter-VLAN communication.

- Simulate a scenario where users in the Shoppers VLAN (Flipkart) cannot ping users in the Streamers VLAN (Spotify). Identify and fix the misconfiguration causing the issue, then verify connectivity.<br><br><em><strong>Hint:</strong> Check VLAN assignments, router sub-interface configurations, and trunk port settings.</em>

Let's deliberately create and then fix a common inter-VLAN connectivity problem between Shoppers (VLAN 20) and Streamers (VLAN 30).

1. Simulate the problem:

Assume:

- PC2 – Shoppers: 192.168.20.10

- PC3 – Streamers: 192.168.30.10

- Router G0/0.20 → 192.168.20.1

- Router G0/0.30 → 192.168.30.1

From PC2 → Command Prompt:

ping 192.168.30.10

❌ The ping should fail.

2. Check VLAN assignments:

On the switch:

show vlan brief

Check that:

VLAN 20 → PC2's port

VLAN 30 → PC3's port

For example:

20   Shoppers    active    Fa0/3

30   Streamers   active    Fa0/4

If PC3's port is incorrectly assigned to VLAN 20, fix it:

enable

configure terminal

interface fastethernet 0/4

switchport mode access

switchport access vlan 30

no shutdown

end

3. Check the router sub-interfaces:

On the router:

show ip interface brief

You should have:

G0/0.20    192.168.20.1    up    up

G0/0.30    192.168.30.1    up    up

If G0/0.30 is missing or incorrectly configured, fix it:

enable

configure terminal

interface gigabitethernet 0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

end

Make sure the physical interface is enabled:

configure terminal

interface gigabitethernet 0/0

no shutdown

end

4. Check the switch-to-router trunk:

On the switch:

show interfaces trunk

The port connected to the router should appear as a trunk.

For example, if the router is connected to Fa0/1:

interface fastethernet 0/1

switchport mode trunk

no shutdown

end

Make sure VLANs 20 and 30 are allowed across the trunk. You can explicitly configure:

interface fastethernet 0/1

switchport trunk allowed vlan 20,30

If your setup also needs VLAN 10:

switchport trunk allowed vlan 10,20,30

5. Check PC3's IP configuration:

On PC3 → Desktop → IP Configuration:

IP Address:       192.168.30.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.30.1

PC2 should have:

IP Address:       192.168.20.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.20.1

6. Verify the fix:

First test PC2's gateway:

ping 192.168.20.1

✅ Should succeed.

Then PC2 → PC3:

ping 192.168.30.10

✅ Should now succeed.

You can also test the reverse direction from PC3:

ping 192.168.20.10

✅ Should succeed.

Final diagnosis:

A typical misconfiguration could be:

PC3

↓

Fa0/4 incorrectly assigned to VLAN 20

instead of:

PC3

↓

Fa0/4 → VLAN 30

↓

Switch trunk

↓

Router G0/0.30 → 192.168.30.1

Conclusion: After correcting the VLAN assignment, ensuring the trunk carries VLAN 30, and verifying the router's G0/0.30 sub-interface, Shoppers (Flipkart) and Streamers (Spotify) can successfully ping each other.

- Compare the router-on-a-stick method you just configured with inter-VLAN routing using a Layer 3 switch: list two advantages and two disadvantages of each approach in a table.

| Method: | Advantages | Disadvantages |
| --- | --- | --- |
| Router-on-a-Stick: | Low cost. | Can become a bottleneck. |
|  | Easy to configure. | Router failure stops inter-VLAN routing. |
| Layer 3 Switch: | Faster routing. | More expensive. |
|  | Good for large networks. | More complex to configure. |

THE END…..