~VLAN Concept Theory:-

- On a Cisco Packet Tracer switch, create two VLANs named 'Engineering' (VLAN 10) and 'Marketing' (VLAN 20) using the CLI commands.

Cisco Packet Tracer: Create Engineering and Marketing VLANs

1. Open the Switch CLI:

Click Switch → CLI and enter:

enable

configure terminal

2. Create VLAN 10 — Engineering:

vlan 10

name Engineering

exit

3. Create VLAN 20 — Marketing:

vlan 20

name Marketing

exit

4. Verify the VLANs:

show vlan brief

You should see something similar to:

VLAN Name                             Status    Ports

---- -------------------------------- --------- ----------------

1    default                          active

10   Engineering                      active

20   Marketing                        active

Result

| VLAN ID | VLAN Name | Purpose |
| --- | --- | --- |
| 10 | Engineering | Engineering department |
| 20 | Marketing | Marketing department |

Conclusion: VLAN 10 and VLAN 20 have been successfully created on the Cisco switch. They are currently empty until switch ports are assigned to them.

- Assign ports FastEthernet0/1 and FastEthernet0/2 to VLAN 10, and ports FastEthernet0/3 and FastEthernet0/4 to VLAN 20 on your switch.

Assign Switch Ports to VLANs:

Use the same switch where you created VLAN 10 – Engineering and VLAN 20 – Marketing.

1. Assign Fa0/1 and Fa0/2 to VLAN 10:

enable

configure terminal

interface range fastEthernet 0/1 - 2

switchport mode access

switchport access vlan 10

exit

2. Assign Fa0/3 and Fa0/4 to VLAN 20:

interface range fastEthernet 0/3 - 4

switchport mode access

switchport access vlan 20

exit

3. Save the configuration:

end

copy running-config startup-config

Press Enter if Packet Tracer asks for the destination filename.

4. Verify:

Run:

show vlan brief

You should see:

VLAN Name                             Status    Ports

---- -------------------------------- --------- ----------------

1    default                          active    Fa0/5, Fa0/6...

10   Engineering                      active    Fa0/1, Fa0/2

20   Marketing                        active    Fa0/3, Fa0/4

Final assignment:

| Ports | VLAN | Department |
| --- | --- | --- |
| Fa0/1 | 10 | Engineering |
| Fa0/2 | 10 | Engineering |
| Fa0/3 | 20 | Marketing |
| Fa0/4 | 20 | Marketing |

Result: Fa0/1–Fa0/2 are members of Engineering VLAN 10, while Fa0/3–Fa0/4 are members of Marketing VLAN 20.

- Write a short document (in a text file) listing the VLAN IDs, names, and the ports assigned to each VLAN from your configuration.

from pathlib import Path

content = """VLAN Configuration Summary

VLAN 10 – Engineering

Assigned Ports:

- FastEthernet0/1

- FastEthernet0/2

VLAN 20 – Marketing

Assigned Ports:

- FastEthernet0/3

- FastEthernet0/4 "

"" 
path = Path("/mnt/data/VLAN_Configuration_Summary.txt") path.write_text(content, encoding="utf-8")

print(path)

- Explain, in your own words, two benefits of using VLANs in a college campus network that has separate labs for coding and networking.

Two Benefits of VLANs in a College Campus:

- Better security: VLANs can separate the coding lab and networking lab into different logical networks. Students in one lab cannot directly access devices in the other lab unless the network administrator allows it.

- Less network traffic: VLANs divide the campus network into smaller broadcast domains. For example, broadcasts from the coding lab stay within its VLAN instead of unnecessarily reaching all computers in the networking lab. This can make the network more organized and efficient.

THE END……