~Day-2:

- List three differences between static routing and dynamic routing in a table format, using real-world analogies from apps like Google Maps or Swiggy to make each point relatable.

Static Routing vs Dynamic Routing: —

1. Route Configuration:

- Static Routing: The administrator manually enters the route into the router.

- Dynamic Routing: The router automatically learns routes using routing protocols.

- Analogy: Static routing is like manually choosing a fixed route in Google Maps, while dynamic routing is like Google Maps automatically selecting the best route.

2. Route Failure:

- Static Routing: If the configured route fails, the administrator must usually change the route manually.

- Dynamic Routing: If a route fails, the router can automatically find another available route.

- Analogy: If a road is blocked for a Swiggy delivery, static routing needs manual instructions, while dynamic routing can automatically choose another road.

3. Network Management:

- Static Routing: It is easier to manage in small networks because there are only a few routes to configure.

- Dynamic Routing: It is more suitable for large networks because routers automatically exchange and update routing information.

- Analogy: A small local delivery service can use fixed routes, while a large service like Swiggy needs routes that can adapt to changing roads and traffic.

- Open Cisco Packet Tracer and load a pre-configured network file with three routers using RIP. Use the 'show ip route' and 'show ip protocols' commands on each router to observe and write down the RIP-learned routes and their metrics.

Cisco Packet Tracer – Observe RIP Routes and Metrics:

- Open Cisco Packet Tracer on your computer.

- Open the pre-configured Packet Tracer file containing three routers configured with RIP.

- Make sure the three routers are connected and powered on.

- Click Router1 → CLI.

- Enter:

enable

show ip route

- Look for routes marked with R.
R = RIP-learned route.

- Write down the RIP route, destination network, next-hop address, and metric.
Example:    R    192.168.3.0/24 [120/2] via 10.0.0.2

- Here, 120 is the RIP administrative distance and 2 is the RIP metric (hop count).

- Now enter:

show ip protocols

- This command shows that RIP is running, the networks participating in RIP, and information about RIP updates.

- Repeat the same commands on Router2:

enable

show ip route

show ip protocols

- Record all routes beginning with R and their hop-count metrics.

- Repeat the commands on Router3:

enable

show ip route

show ip protocols

Important Point:

RIP uses hop count as its metric. A route with 1 hop is generally preferred over a route with 2 hops. RIP supports a maximum of 15 hops; 16 hops means the destination is unreachable.

| A: | B: | C: |
| --- | --- | --- |
| Show sample show ip route output | Explain RIP metrics | Give Packet Tracer steps with screenshots |

- Simulate a network change by shutting down one interface on a router in the RIP-enabled topology. Observe and record how long it takes for the routing tables on the other routers to update and remove the unreachable route.<br><br><em><strong>Hint:</strong> Use the 'shutdown' command on an interface and monitor with 'show ip route' repeatedly.</em>

RIP Network Change Simulation:

1. Check the current routing table:

On Router1, open the CLI and enter:

enable

show ip route

Find the RIP route marked with R and note the destination network.

2. Shut down an interface:

Suppose the interface connecting Router1 to Router2 is Serial0/0/0.

On Router1, enter:

enable

configure terminal

interface serial 0/0/0

shutdown

The interface should change to administratively down.

3. Monitor Router2:

Immediately go to Router2 and repeatedly enter:

show ip route

Look for the RIP route that was reachable through Router1.

Initially, the route may still appear because RIP does not remove failed routes immediately.

4. Wait for RIP to update:

Continue checking:

show ip route

After RIP detects the failure and updates the routing information, the unreachable route should disappear or be replaced by another available route.

You can also check RIP information with:

show ip protocols

5. Record the observation:

| Observation | Result |
| --- | --- |
| 1.)Interface shut down | Serial0/0/0 |
| 2.)Affected router | Router1 |
| 3.)Other router monitored | Router2 |
| 4.)Initial RIP route | Present |
| 5.)Route after RIP update | Removed/replaced |

Conclusion:

When an interface is shut down, the directly connected path becomes unavailable. RIP detects the topology change, exchanges updated routing information, and the other routers eventually remove the unreachable route or select an alternative route.

- Explain in 3-4 sentences how dynamic routing in apps like Uber or Zomato delivery tracking is similar to RIP dynamic routing in networks. Focus on how routes or paths are updated automatically in both cases.

Dynamic Routing in RIP vs Uber/Zomato:

- RIP dynamic routing automatically exchanges routing information between routers and updates the routing table when the network changes.

- Similarly, Uber or Zomato can automatically update a driver's or delivery partner's route when traffic, road closures, or other conditions change.

- In RIP, routers can select another available path when the original path becomes unavailable; in delivery apps, the system can suggest an alternative road.

- The main similarity is that both systems automatically adapt to changes, reducing the need for manual route changes and helping data or deliveries reach the destination efficiently.

THE END…..