~Day-3:

- Set up a network topology with three routers in Cisco Packet Tracer, connect them using serial or Ethernet links, and assign IP addresses to all interfaces.

Cisco Packet Tracer: 3-Router Network Setup:

1. Create the topology:

Open Cisco Packet Tracer and add:

- 3 Routers: Router1, Router2, Router3

- 3 PCs: PC1, PC2, PC3

- Connect them like this:

PC1 ─── Router1 ─── Router2 ─── Router3 ─── PC3

│

PC2

For a simple setup, use Ethernet links between the routers.

2. IP Addressing Plan:

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Router1 | G0/0 | 192.168.1.1 | 255.255.255.0 |
| Router1 | G0/1 | 10.0.12.1 | 255.255.255.252 |
| Router2 | G0/0 | 10.0.12.2 | 255.255.255.252 |
| Router2 | G0/1 | 10.0.23.1 | 255.255.255.252 |
| Router3 | G0/0 | 10.0.23.2 | 255.255.255.252 |
| Router3 | G0/1 | 192.168.3.1 | 255.255.255.0 |

PC configuration:

| PC | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC3 | 192.168.3.10 | 255.255.255.0 | 192.168.3.1 |

If your router model has different interface names, such as Fa0/0 instead of G0/0, use the interface available on your router.

3. Configure Router1:

Click Router1 → CLI:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

end

4. Configure Router2:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

end

5. Configure Router3:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

end

6. Verify the interfaces:

On each router, run:

show ip interface brief

You should see the configured interfaces as:

Status: up

Protocol: up

7. Configure the PCs:

Go to:

PC → Desktop → IP Configuration

Enter the corresponding IP address, subnet mask, and default gateway from the table above.

8. Test the network:

From PC1 → Desktop → Command Prompt, test:

ping 192.168.1.1

This checks PC1 → Router1.

Then later, after configuring routing, you can test:

ping 192.168.3.10

This checks communication from PC1 to PC3 through all three routers.

Result: You now have a three-router topology with IP addresses assigned to all router interfaces and PCs, ready for configuring RIP, static routing, or another routing protocol.

- Enable RIP version 2 on all three routers in your topology, advertise all directly connected networks, and verify that each router learns routes to all subnets using the 'show ip route' command.

Enable RIP Version 2 on All Three Routers:

Using the IP addressing from the previous topology, configure RIPv2 on each router.

1. Configure Router1:

Open Router1 → CLI:

enable

configure terminal

router rip

version 2

no auto-summary

network 192.168.1.0

network 10.0.12.0

exit

end

Router1 advertises:

- 192.168.1.0/24 — PC1 LAN

- 10.0.12.0/30 — Router1 ↔ Router2 link

2. Configure Router2:

Open Router2 → CLI:

enable

configure terminal

router rip

version 2

no auto-summary

network 10.0.12.0

network 10.0.23.0

exit

end

Router2 advertises:

- 10.0.12.0/30 — Router1 ↔ Router2 link

- 10.0.23.0/30 — Router2 ↔ Router3 link

3. Configure Router3:

Open Router3 → CLI:

enable

configure terminal

router rip

version 2

no auto-summary

network 10.0.23.0

network 192.168.3.0

exit

end

Router3 advertises:

- 10.0.23.0/30 — Router2 ↔ Router3 link

- 192.168.3.0/24 — PC3 LAN

4. Verify RIP Configuration:

On each router, run:

show ip protocols

You should see:

Routing Protocol is "rip"

Sending updates every 30 seconds

Outgoing update filter list for all interfaces is not set

Routing for Networks:

This confirms that RIP is enabled.

5. Check the Routing Table:

On Router1:

show ip route

Look for routes beginning with R.

For example:

R    10.0.23.0/30 [120/1] via 10.0.12.2

R    192.168.3.0/24 [120/2] via 10.0.12.2

Here, R means the route was learned through RIP, 120 is RIP's administrative distance, and the final number is the hop-count metric.

Router2

show ip route

You should see RIP routes such as:

R    192.168.1.0/24 [120/1] via 10.0.12.1

R    192.168.3.0/24 [120/1] via 10.0.23.2

Router3

show ip route

You should see:

R    10.0.12.0/30 [120/1] via 10.0.23.1

R    192.168.1.0/24 [120/2] via 10.0.23.1

6. Final Verification:

| Router | Directly Connected Networks | RIP-Learned Networks |
| --- | --- | --- |
| Router1 | 192.168.1.0/24, 10.0.12.0/30 | 10.0.23.0/30, 192.168.3.0/24 |
| Router2 | 10.0.12.0/30, 10.0.23.0/30 | 192.168.1.0/24, 192.168.3.0/24 |
| Router3 | 10.0.23.0/30, 192.168.3.0/24 | 10.0.12.0/30, 192.168.1.0/24 |

Conclusion: RIPv2 has been enabled on all three routers. Each router advertises its directly connected networks and automatically learns the remote networks from neighboring routers. Routes marked with R in show ip route confirm that they were learned through RIP.

- Change the hop count metric for one of the networks by adding an extra router between two existing routers, then observe and record how the hop count value changes in the routing tables.

Change RIP Hop Count by Adding an Extra Router:

1. Original Topology:

Initially, the network has three routers:

PC1 — Router1 — Router2 — Router3 — PC3

For example, Router1 reaches the 192.168.3.0/24 network through Router2.

Router1 → Router2 → Router3

1       2

So the RIP metric shown on Router1 may be:

R 192.168.3.0/24 [120/2] via 10.0.12.2

Here, 2 = RIP hop count.

2. Add an Extra Router:

Add Router4 between Router2 and Router3.

New topology:

PC1 — R1 — R2 — R4 — R3 — PC3

The path from Router1 to the PC3 network is now:

Router1 → Router2 → Router4 → Router3

Therefore, the destination is now one router farther away.

3. Configure Router4:

Assign IP addresses to the two interfaces of Router4.

Example:

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| R4 G0/0 | 10.0.24.1 | 255.255.255.252 |
| R4 G0/1 | 10.0.34.1 | 255.255.255.252 |

Configure:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.24.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.34.1 255.255.255.252

no shutdown

exit

Then configure the neighboring Router2 and Router3 interfaces with the corresponding IP addresses.

4. Enable RIPv2 on Router4:

router rip

version 2

no auto-summary

network 10.0.24.0

network 10.0.34.0

exit

Also update the RIP configuration on Router2 and Router3 to advertise their new links.

5. Check the Routing Table:

On Router1, enter:

show ip route

Before adding Router4, you might have observed:

R 192.168.3.0/24 [120/2] via 10.0.12.2

After adding Router4, you should observe a higher metric, such as:

R 192.168.3.0/24 [120/3] via 10.0.12.2

The exact output depends on your topology and interface addressing.

6. Observation:

| Network | Before Adding R4 | After Adding R4 |
| --- | --- | --- |
| 192.168.3.0/24 | 2 hops | 3 hops |
| Path | R1 → R2 → R3 | R1 → R2 → R4 → R3 |

Conclusion:

Adding Router4 creates an additional router hop between Router1 and Router3. Therefore, the RIP hop-count metric increases by 1 for routes that must pass through the additional router. This demonstrates that RIP uses the number of routers (hops) in a path as its routing metric.

- Use the simulation mode in Cisco Packet Tracer to capture and analyze RIP update packets as they are exchanged between routers.<br><br><em><strong>Hint:</strong> Filter for RIP packets in the simulation panel and note the source, destination, and advertised routes in each update.</em>

Cisco Packet Tracer – Capture and Analyze RIP Update Packets:

1. Open the Network:

Open your three-router RIP-enabled topology in Cisco Packet Tracer.

Example:

PC1 — Router1 — Router2 — Router3 — PC3

Make sure RIPv2 is already configured on all routers.

2. Switch to Simulation Mode:

At the bottom-right of Packet Tracer, click:

Simulation

This changes Packet Tracer from Realtime mode to Simulation mode.

3. Filter RIP Packets:

- In the Simulation Panel, click Edit Filters.

- Click None to clear the existing filters.

- Select RIP.

- Make sure other protocols are unchecked if you only want to observe RIP.

Now Packet Tracer will display RIP packets as they are generated.

4. Generate/Wait for RIP Updates:

RIP sends routing updates periodically.

Click Capture/Forward in the Simulation Panel to move through the network events one at a time.

When a RIP packet appears:

- Click the envelope/packet.

- Open the PDU Details or Inbound/Outbound PDU Details.

- Examine the RIP information.

5. Record the RIP Packet Information:

Create a table like this:

| Packet | Source Router | Destination | Advertised Network | Metric |
| --- | --- | --- | --- | --- |
| 1 | Router1 | Router2 | 192.168.1.0/24 | 1 |
| 2 | Router2 | Router1 | 192.168.3.0/24 | 1 |
| 3 | Router2 | Router3 | 192.168.1.0/24 | 1 |
| 4 | Router3 | Router2 | 192.168.3.0/24 | 1 |

Use the actual values displayed in your Packet Tracer simulation because they depend on your topology.

6. Analyze the Packet:

For each RIP update, note:

Source: The router sending the RIP update.

Destination: The neighboring router receiving the update. RIPv2 uses multicast address 224.0.0.9 for updates.

Advertised Route: The network information being shared with the neighboring router.

Metric: The RIP hop count associated with the advertised network.

7. Observe Route Learning:

For example:

Router1 → Router2

Router1 may advertise:

192.168.1.0/24

Router2 receives the update and can add that network to its routing table.

Then Router2 can advertise the learned network toward Router3:

Router2 → Router3

192.168.1.0/24

The hop count increases as the route passes through routers.

8. Verify the Routing Table:

After observing the packets, switch back to the router CLI and enter:

show ip route

Look for routes beginning with:

R

R indicates that the route was learned through RIP.

You can also use:

show ip protocols

to verify that RIPv2 is running.

Observation/Result:

RIP update packets were successfully captured in Simulation Mode. The packets showed the source router, destination, advertised networks, and RIP metrics. The routers exchanged routing information with their neighbors, allowing each router to learn routes to remote networks automatically.

THE END………..