~VLAN Trunking Access Port Theory:-

- In Cisco Packet Tracer, configure two switches (Switch1 and Switch2) and connect them with a crossover cable. Set the FastEthernet0/1 port on both switches as trunk ports using the 802.1Q protocol.

Here are the exact Cisco Packet Tracer steps and commands to configure Switch1 and Switch2 with a trunk link on FastEthernet0/1 using IEEE 802.1Q.

1. Create the topology:

Add:

- 2 × 2960 switches

- Connect Switch1 Fa0/1 ↔ Switch2 Fa0/1

- Use a Copper Cross-Over cable.

2. Configure Switch1:

Click Switch1 → CLI and enter:

enable

configure terminal

interface fastethernet 0/1

switchport mode trunk

no shutdown

exit

end

3. Configure Switch2:

Click Switch2 → CLI:

enable

configure terminal

interface fastethernet 0/1

switchport mode trunk

no shutdown

exit

end

4. Verify the trunk:

On both switches, run:

show interfaces trunk

You should see Fa0/1 listed as a trunk port.

You can also check:

show interfaces fastethernet 0/1 switchport

Look for:

Administrative Mode: trunk

Operational Mode: trunk

Important note about 802.1Q:

On many Cisco 2960 switches in Packet Tracer, the trunk uses 802.1Q automatically when you configure:

switchport mode trunk

Older Cisco switch models may require:

switchport trunk encapsulation dot1q

If Packet Tracer accepts that command, you can use it before switchport mode trunk. If it says the command is unavailable, that's normal for models where 802.1Q is the only supported trunk encapsulation.

Expected result:

Switch1 Fa0/1  ======= TRUNK (802.1Q) =======  Fa0/1 Switch2

The link should become green, and show interfaces trunk should confirm the trunk is active.

- Create three VLANs (10, 20, 30) on both switches and assign FastEthernet0/2, 0/3, and 0/4 on Switch1 to VLAN 10, 20, and 30 respectively as access ports.

Since Switch1 and Switch2 are already connected through Fa0/1 as a trunk, now create VLANs 10, 20, 30 on both switches and assign the ports on Switch1.

1. Configure VLANs on Switch1:

Open Switch1 → CLI:

enable

configure terminal

vlan 10

name VLAN10

exit

vlan 20

name VLAN20

exit

vlan 30

name VLAN30

exit

2. Assign Switch1 ports;

Fa0/2 → VLAN 10

interface fastethernet 0/2

switchport mode access

switchport access vlan 10

no shutdown

exit

Fa0/3 → VLAN 20

interface fastethernet 0/3

switchport mode access

switchport access vlan 20

no shutdown

exit

Fa0/4 → VLAN 30

interface fastethernet 0/4

switchport mode access

switchport access vlan 30

no shutdown

exit

Then:

end

3. Create the same VLANs on Switch2;

Open Switch2 → CLI:

enable

configure terminal

vlan 10

name VLAN10

exit

vlan 20

name VLAN20

exit

vlan 30

name VLAN30

exit

end

Don't assign Switch2 Fa0/2–Fa0/4 yet unless your next task asks you to.

4. Verify VLANs

On both switches:

show vlan brief

On Switch1, you should see approximately:

VLAN Name       Status    Ports

10   VLAN10     active    Fa0/2

20   VLAN20     active    Fa0/3

30   VLAN30     active    Fa0/4

And verify the trunk is still working:

show interfaces trunk

Final topology:

TRUNK

Fa0/1 ================= Fa0/1

Switch1                 Switch2

|

-----------------

|       |       |

Fa0/2   Fa0/3   Fa0/4

|       |       |

VLAN 10  VLAN 20  VLAN 30

Result: VLANs 10, 20, and 30 exist on both switches, while Switch1's Fa0/2, Fa0/3, and Fa0/4 are configured as access ports for VLANs 10, 20, and 30, respectively.

- Connect three PCs to Switch1 on ports assigned to VLAN 10, 20, and 30. Verify that PCs in different VLANs cannot communicate with each other, but can communicate with other PCs in the same VLAN.

1. Connect the PCs:

In Cisco Packet Tracer, add 3 PCs and connect them to Switch1 using Copper Straight-Through cables:

| PC | Switch1 Port | VLAN |
| --- | --- | --- |
| PC1 | Fa0/2 | VLAN 10 |
| PC2 | Fa0/3 | VLAN 20 |
| PC3 | Fa0/4 | VLAN 30 |

2. Configure IP addresses:

Since each PC is in a different VLAN, use a different subnet for each:

PC1 — VLAN 10

IP Address: 192.168.10.10

Subnet Mask: 255.255.255.0

PC2 — VLAN 20

IP Address: 192.168.20.10

Subnet Mask: 255.255.255.0

PC3 — VLAN 30

IP Address: 192.168.30.10

Subnet Mask: 255.255.255.0

You don't need to configure a default gateway for this test because there is no router providing inter-VLAN routing.

3. Verify VLAN assignment;

On Switch1, run:

show vlan brief

You should see:

VLAN 10 → Fa0/2

VLAN 20 → Fa0/3

VLAN 30 → Fa0/4

4. Test communication:

Open PC1 → Desktop → Command Prompt.

Test PC2:

ping 192.168.20.10

❌ Should fail because PC1 is in VLAN 10 and PC2 is in VLAN 20.

Test PC3:

ping 192.168.30.10

❌ Should fail because PC1 is in VLAN 10 and PC3 is in VLAN 30.

5. Test same-VLAN communication

To properly test same-VLAN communication, add another PC to Switch1, for example:

PC4 → Fa0/5

Configure Fa0/5 as VLAN 10:

enable

configure terminal

interface fastethernet 0/5

switchport mode access

switchport access vlan 10

no shutdown

end

Configure PC4:

IP Address: 192.168.10.11

Subnet Mask: 255.255.255.0

From PC1:

ping 192.168.10.11

✅ Should succeed, because both PCs are in VLAN 10 and the same IP subnet.

Expected result:

| Test | Expected |
| --- | --- |
| PC1 → PC4 (VLAN 10 → VLAN 10) | ✅ Success |
| PC1 → PC2 (VLAN 10 → VLAN 20) | ❌ Fail |
| PC1 → PC3 (VLAN 10 → VLAN 30) | ❌ Fail |
| PC2 → PC3 (VLAN 20 → VLAN 30) | ❌ Fail |

Conclusion: VLANs logically separate devices into different broadcast domains. Devices in different VLANs cannot communicate directly through a Layer-2 switch. Inter-VLAN routing using a router or Layer-3 switch would be required for communication between VLANs.

- Change the trunk encapsulation on the trunk link between Switch1 and Switch2 from 802.1Q to ISL, and test if VLAN traffic still passes between the switches.<br><br><em><strong>Hint:</strong> Use the 'switchport trunk encapsulation isl' command before setting the port to trunk mode.</em>

In Cisco Packet Tracer, this task depends on the switch model. Most 2960 switches do not support ISL; they support 802.1Q only. So the command may produce an error.

1. Try ISL on Switch1:

enable

configure terminal

interface fastethernet 0/1

switchport trunk encapsulation isl

switchport mode trunk

end

2. Try ISL on Switch2:

enable

configure terminal

interface fastethernet 0/1

switchport trunk encapsulation isl

switchport mode trunk

end

3. Check the result:

Run on both switches:

show interfaces trunk

Also check:

show interfaces fastethernet 0/1 switchport

If you get an error:

On a typical Cisco 2960 in Packet Tracer, you may see something like:

% Command rejected: Fa0/1 does not support ISL encapsulation

This is expected because 2960 switches don't support ISL.

Therefore, the VLAN traffic will not be carried using ISL on these switches. The trunk must remain 802.1Q.

Restore 802.1Q trunk:

If necessary, simply configure:

enable

configure terminal

interface fastethernet 0/1

switchport mode trunk

no shutdown

end

Then verify:

show interfaces trunk

Practical conclusion:

Result: ISL cannot be configured on the commonly used Packet Tracer 2960 switches because they support 802.1Q trunking only. Therefore, VLAN traffic continues to pass between Switch1 and Switch2 using 802.1Q, but ISL cannot be tested on this switch model.

- Simulate a scenario where a new port on Switch2 (FastEthernet0/5) is accidentally set as a trunk port instead of an access port for VLAN 20. Observe and explain the impact on connectivity for a PC connected to that port.

1. Accidentally configure Fa0/5 as a trunk:

On Switch2 → CLI:

enable

configure terminal

interface fastethernet 0/5

switchport mode trunk

no shutdown

end

Verify:

show interfaces trunk

You should see Fa0/5 listed as a trunk.

2. Connect a PC to Fa0/5:

Connect PC4 → Switch2 Fa0/5 using a Copper Straight-Through cable.

Give PC4 an address from VLAN 20's network, for example:

IP Address: 192.168.20.11

Subnet Mask: 255.255.255.0

If there is a gateway configured for VLAN 20, use the appropriate gateway.

3. Test connectivity:

From PC4:

ping 192.168.20.10

where 192.168.20.10 is the IP of another PC in VLAN 20.

Because Fa0/5 is configured as a trunk, the PC is connected to a port intended to carry tagged VLAN traffic, rather than a normal access port that places incoming untagged PC frames into VLAN 20.

Therefore, PC4 may not communicate correctly with VLAN 20 devices.

4. Correct the mistake:

Change Fa0/5 back to an access port and assign it to VLAN 20:

enable

configure terminal

interface fastethernet 0/5

switchport mode access

switchport access vlan 20

no shutdown

end

Verify:

show vlan brief

You should now see:

20   VLAN20   active   Fa0/5

Test again:

ping 192.168.20.10

✅ The PC should now communicate with other devices in VLAN 20.

Explanation:

| Configuration | PC connectivity |
| --- | --- |
| Fa0/5 as Trunk | ❌ PC may not communicate correctly |
| Fa0/5 as Access VLAN 20 | ✅ PC communicates normally |

Conclusion: A trunk port is designed primarily for connections between network devices and carries multiple VLANs. A PC should normally be connected to an access port, which assigns its traffic to a specific VLAN. Accidentally configuring the PC's port as a trunk can cause connectivity problems because the PC normally sends and receives untagged Ethernet frames.

THE END…..