~Day-4:

- Open Cisco Packet Tracer and create a simple network with three routers connected in a triangle topology using serial or Ethernet links.

1. Add devices

Open Cisco Packet Tracer and place:

- 3 Routers — use 2911 or 1941

- Connect them in a triangle:

R1

/  \

/    \

R2 ------ R3

2. Connect the routers

For an easy setup, use Ethernet/GigabitEthernet links:

| Link | Interface 1 | Interface 2 | Network |
| --- | --- | --- | --- |
| R1 ↔ R2 | R1 G0/0 | R2 G0/0 | 10.0.12.0/30 |
| R2 ↔ R3 | R2 G0/1 | R3 G0/0 | 10.0.23.0/30 |
| R3 ↔ R1 | R3 G0/1 | R1 G0/1 | 10.0.31.0/30 |

Use the Copper Cross-Over cable if Packet Tracer doesn't automatically select the connection.

3. Configure R1

Click R1 → CLI:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.31.1 255.255.255.252

no shutdown

exit

end

4. Configure R2

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

end

5. Configure R3

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.31.2 255.255.255.252

no shutdown

exit

end

6. Check the connections

On each router:

show ip interface brief

You should see the connected interfaces as up/up.

Test directly connected routers:

From R1:

ping 10.0.12.2

ping 10.0.31.2

From R2:

ping 10.0.23.2

At this stage, the three routers are physically connected in a triangle. Routing protocols or static routes are not yet required just to create the topology.

- Assign IP addresses to all router interfaces in your Packet Tracer network and verify connectivity using the ping command.

IP Addressing:

| Router | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/0 | 10.0.12.1 | 255.255.255.252 |
| R1 | G0/1 | 10.0.31.1 | 255.255.255.252 |
| R2 | G0/0 | 10.0.12.2 | 255.255.255.252 |
| R2 | G0/1 | 10.0.23.1 | 255.255.255.252 |
| R3 | G0/0 | 10.0.23.2 | 255.255.255.252 |
| R3 | G0/1 | 10.0.31.2 | 255.255.255.252 |

R1 Configuration

enable

configure terminal

interface g0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.31.1 255.255.255.252

no shutdown

exit

end

R2 Configuration

enable

configure terminal

interface g0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

end

R3 Configuration

enable

configure terminal

interface g0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

interface g0/1

ip address 10.0.31.2 255.255.255.252

no shutdown

exit

end

Verify interfaces

On each router:

show ip interface brief

You should see up/up for all six interfaces.

Test connectivity with Ping

From R1:

ping 10.0.12.2

ping 10.0.31.2

From R2:

ping 10.0.12.1

ping 10.0.23.2

From R3:

ping 10.0.23.1

ping 10.0.31.1

If everything is configured correctly, you should receive:

!!!!!.......This means the directly connected router interfaces can communicate successfully.

- Configure OSPF on each router using the router ospf 1 command and the network command to advertise all directly connected networks.

Using the same 3-router triangle topology, configure OSPF Process ID 1 on all routers:

Network topology:

R1

/      \

/        \

10.0.12.0      10.0.31.0

/            \

R2------------R3

10.0.23.0

1. Configure OSPF on R1

enable

configure terminal

router ospf 1

network 10.0.12.0 0.0.0.3 area 0

network 10.0.31.0 0.0.0.3 area 0

end

2. Configure OSPF on R2

enable

configure terminal

router ospf 1

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

end

3. Configure OSPF on R3

enable

configure terminal

router ospf 1

network 10.0.23.0 0.0.0.3 area 0

network 10.0.31.0 0.0.0.3 area 0

end

4. Verify OSPF neighbors

On each router:

show ip ospf neighbor

You should see the other connected routers as FULL neighbors.

For example, R1 should form OSPF neighbor relationships with R2 and R3.

5. Check the routing table

Run:

show ip route

Look for routes marked with O:

O    10.0.23.0/30

O means the route was learned through OSPF.

6. Test end-to-end connectivity

From R1:

ping 10.0.23.2

From R2:

ping 10.0.31.2

From R3:

ping 10.0.12.1

If OSPF is working correctly, the pings should succeed.

Important: The 0.0.0.3 values are OSPF wildcard masks corresponding to the /30 networks you configured earlier.

- Observe and document the OSPF adjacency formation process by checking the output of the show ip ospf neighbor command on each router.<br><br><em><strong>Hint:</strong> Look for the FULL/DR or FULL/BDR state to confirm adjacency.</em>

OSPF Adjacency Verification:

After configuring OSPF on the three routers, check the neighbor relationships using:

show ip ospf neighbor

R1:

On R1:

R1# show ip ospf neighbor

You should see neighbors R2 and R3 with a state such as:

Neighbor ID     State

2.2.2.2         FULL/DR

3.3.3.3         FULL/BDR

R2:

R2# show ip ospf neighbor

Expected:

Neighbor ID     State

1.1.1.1         FULL/DR

3.3.3.3         FULL/BDR

R3:

R3# show ip ospf neighbor

Expected:

Neighbor ID     State

1.1.1.1         FULL/DR

2.2.2.2         FULL/BDR

Note: The exact DR/BDR designation can differ depending on OSPF router IDs and interface priorities. Don't worry if your DR/BDR labels aren't exactly like the example.

What to observe

The important part is the FULL state.

- FULL → OSPF routers have successfully established adjacency.

- DR → Designated Router.

- BDR → Backup Designated Router.

- If you see FULL/DR or FULL/BDR, the adjacency is successfully established.

Documentation for your practical

Observation:
After configuring OSPF on R1, R2, and R3, the show ip ospf neighbor command displayed the neighboring routers in the FULL state. This confirms that OSPF neighbor relationships were successfully established and the routers can exchange OSPF routing information.

Result:
✅ OSPF adjacency successfully formed between the three routers.

- Change the OSPF cost on one interface using the ip ospf cost command and verify how it affects the OSPF routing table.<br><br><em><strong>Hint:</strong> Use show ip route ospf to see route changes after modifying the cost.</em>

Change OSPF Cost and Verify the Routing Table:

Using the same R1–R2–R3 triangle, we can increase the OSPF cost of one link and observe how OSPF chooses a different path.

1. Check the current OSPF routes:

On R1, run:

show ip route ospf

Before changing the cost, you may see a route such as:

O    10.0.23.0/30 [110/2] via 10.0.12.2

Here, 110 is the OSPF administrative distance and 2 is the OSPF metric/cost.

2. Increase the cost on R1's interface:

Let's make the R1 → R2 link expensive.

On R1:

enable

configure terminal

interface gigabitEthernet 0/0

ip ospf cost 100

exit

end

This changes the OSPF cost of the R1–R2 interface to 100.

3. Wait for OSPF to recalculate:

Give Packet Tracer a few seconds, then check:

show ip route ospf

You can also check the OSPF neighbor relationship:

show ip ospf neighbor

The neighbor should still normally be FULL. Changing the cost does not normally break the OSPF adjacency.

4. Observe the route change:

Previously, R1 may have preferred:

R1 → R2 → R3

After making R1's R2 link cost 100, the direct R1–R3 path becomes more attractive:

R1 → R3

So the routing table may change from a route through R2 to a route through R3, depending on the complete topology and costs.

Check it with:

show ip route ospf

You can also use:

show ip ospf interface gigabitEthernet 0/0

to confirm that the interface cost is 100.

Practical Observation:

Before: R1 preferred the lower-cost path through R2.

After: Increasing the R1–R2 interface cost to 100 caused OSPF to recalculate its shortest path. If the alternative R1–R3 path has a lower total cost, R1 will select that path instead.

Result: ✅ OSPF automatically recalculated the routing path based on the changed interface cost.

THE END……….