~Day-1:

- Open Cisco Packet Tracer and set up a simple network with two routers connected via a serial cable, assigning IP addresses to both router interfaces.

1. Place the routers:

- Open Cisco Packet Tracer.

- Go to Network Devices → Routers.

- Drag 2 routers into the workspace.

- Rename them Router0 and Router1.

2. Add serial interfaces:

If the routers don't already have serial ports:

- Click Router0 → Physical.

- Turn the router OFF.

- Add a WIC-2T serial module.

- Turn the router ON.

- Do the same for Router1.

3. Connect the routers:

- Select Connections (⚡).

- Choose Serial DCE cable.

- Connect:

  - Router0 Serial0/0/0

  - Router1 Serial0/0/0

Your topology should look like:

Router0 ───── Serial Cable ───── Router1

4. Configure Router0:

Open Router0 → CLI and enter:

enable

configure terminal

interface serial 0/0/0

ip address 192.168.1.1 255.255.255.252

clock rate 64000

no shutdown

exit

end

5. Configure Router1:

Open Router1 → CLI:

enable

configure terminal

interface serial 0/0/0

ip address 192.168.1.2 255.255.255.252

no shutdown

exit

end

6. Verify the interfaces:

On both routers, run:

show ip interface brief

You should see the Serial interface as:

Interface       IP-Address      Status      Protocol

Serial0/0/0     192.168.1.1    up          up

and on Router1:

Serial0/0/0     192.168.1.2    up          up

7. Test connectivity:

From Router0:

ping 192.168.1.2

If you receive replies, the two routers are successfully connected.

IP Address Summary:

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Router0 | Serial0/0/0 | 192.168.1.1 | 255.255.255.252 |
| Router1 | Serial0/0/0 | 192.168.1.2 | 255.255.255.252 |

- On Router1 in Cisco Packet Tracer, configure a static route so it can reach a PC connected to Router2’s LAN network. Test connectivity using the ping command from Router1 to the PC.

1. Configure Router1:

Open Router1 → CLI:

enable

configure terminal

interface serial 0/0/0

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

2. Configure Router2:

enable

configure terminal

interface serial 0/0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

Configure PC2:

IP Address:     192.168.2.10

Subnet Mask:    255.255.255.0

Default Gateway: 192.168.2.1

3. Add the Static Route on Router1:

This is the main step. Router1 needs to know that the 192.168.2.0/24 network is reachable through Router2.

On Router1:

enable

configure terminal

ip route 192.168.2.0 255.255.255.0 10.0.0.2

exit

The command means:

Destination network: 192.168.2.0/24
Next-hop Router: 10.0.0.2

4. Verify the Static Route:

On Router1:

show ip route

You should see something similar to:

S    192.168.2.0/24 [1/0] via 10.0.0.2

S means Static route.

5. Test with Ping:

From Router1:

ping 192.168.2.10

Successful connectivity should show:

!!!!!

This confirms that Router1 can reach the PC connected to Router2's LAN through the static route.

If ping fails:

Check these commands:

Router1:

show ip interface brief

show ip route

Router2:

show ip interface brief

Also make sure:

- Router interfaces are up/up

- PC2 has the correct IP and gateway

- Router2's LAN interface is 192.168.2.1

- The serial connection uses the correct interfaces

- If Router1 is the DCE side of the serial link, configure a clock rate on that interface.

- Simulate a Zomato-style delivery network: imagine Router1 represents an Ahmedabad branch and Router2 represents a Surat branch. Configure static routes so that devices in each city’s LAN can communicate with each other through the routers.

Zomato Delivery Network Simulation::

Imagine:

- Router1 = Ahmedabad Branch

- Router2 = Surat Branch

- Each branch has its own LAN with delivery staff PCs.

Network Design:

| Device | Interface | IP Address |
| --- | --- | --- |
| Router1 (Ahmedabad) | G0/0 | 192.168.1.1/24 |
| Router1 (Ahmedabad) | S0/0/0 | 10.0.0.1/30 |
| Router2 (Surat) | G0/0 | 192.168.2.1/24 |
| Router2 (Surat) | S0/0/0 | 10.0.0.2/30 |
| PC-A (Ahmedabad) | NIC | 192.168.1.10/24 |
| PC-S (Surat) | NIC | 192.168.2.10/24 |

PC Configuration:

Ahmedabad PC:

IP Address: 192.168.1.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

Surat PC:

IP Address: 192.168.2.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.2.1

Configure Router1 (Ahmedabad):

enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 10.0.0.1 255.255.255.252

clock rate 64000

no shutdown

exit

Static Route on Router1:

ip route 192.168.2.0 255.255.255.0 10.0.0.2

Configure Router2 (Surat)

enable

configure terminal

interface g0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

Static Route on Router2:

ip route 192.168.1.0 255.255.255.0 10.0.0.1

Verify Routes

On both routers:

show ip route

You should see entries marked with S (Static).

Test Connectivity:

From Ahmedabad PC:

ping 192.168.2.10

From Surat PC:

ping 192.168.1.10

Expected result:

!!!!!

Real-world Analogy:

- Ahmedabad branch receives an order.

- Router1 forwards traffic to Router2 through the WAN link.

- Router2 delivers the information to the Surat branch LAN.

- Static routes act like predefined delivery routes between Zomato branches.

This Packet Tracer topology demonstrates inter-LAN communication using static routing between two city branches.

- List two advantages and two disadvantages of static routing compared to dynamic routing, using examples from real-life apps like IRCTC or Swiggy to illustrate your points.<br><br><em><strong>Hint:</strong> Think about how static routes might impact delivery or booking updates in these apps.</em>

Static Routing vs Dynamic Routing:

Imagine IRCTC or Swiggy as a large network where routers must decide where to send data.

Advantages of Static Routing:

- Simple and predictable :

  - Routes are manually configured and don't change automatically.

  - Example: Swiggy could use a fixed route between two specific data centers, making the path predictable.

- Less network overhead :

  - Static routing does not exchange routing updates like dynamic routing.

  - Example: For a small IRCTC branch network, this can reduce unnecessary network traffic and keep the network lightweight.

Disadvantages of Static Routing:

- Manual configuration is required:

  - If the network changes, an administrator must update the routes manually.

  - Example: If an IRCTC server or network link changes, booking information may fail to reach the new server until the route is manually updated.

- Poor scalability and fault recovery :

  - Static routes don't automatically find another path when a link fails.

  - Example: If a Swiggy network connection between two regions goes down, delivery/order updates could be delayed until an administrator changes the route.

Quick Comparison

| Static Routing | Dynamic Routing |
| --- | --- |
| Manually configured | Automatically learns routes |
| Predictable | Adapts to network changes |
| Low routing overhead | More routing overhead |
| Doesn't automatically recover from failures | Can automatically find alternative paths |
| Best for small/stable networks | Better for large/changing networks |

THE END…….